Aegis PV Project Report

AI Driven Predictive Health and Performance Maximizer

Aegis-PV is a proposed solar intelligence platform for homes in Saudi Arabia and the wider GCC. It compares expected solar output with actual output, identifies the likely reason for a loss, and gives the homeowner a clear maintenance action. The main goal is to detect dust, shading, and equipment faults before they cause long periods of low energy production.

The project combines a digital twin, live weather data, inverter readings, and machine learning. Its planned design is hardware agnostic through Modbus and REST connections. This makes the concept suitable for different inverter brands and reduces dependence on one vendor ecosystem.

## Main conclusion

Aegis-PV addresses a clear gap between simple residential monitoring tools and advanced enterprise analytics. Its strongest proposed advantage is the combination of GCC climate calibration, residential access, and plain-language maintenance guidance. The next step is to build and test a minimum viable product with local solar data.

## Project team

| Team | SMANERGY |
| --- | --- |
| Team leader | Mohammed Alsuhaibani 202245800 |
| Members | Abdelrahman Elshenawy 202269160 ¦ Mazen Osama 202277380 |
| Members | Abdullah Ewidah 202244620 ¦ Kamal Alkatheri 202044180 |

# Problem and GCC context

Residential solar systems can produce less energy than expected even when the inverter is still running. In the GCC, dust and high temperature can create frequent losses. A homeowner may see lower production but may not know whether the cause is dust, shade, hot weather, a weak string, or an inverter fault. This uncertainty delays the correct response.

## Why the problem matters

·       Dust blocks part of the sunlight that should reach the solar cells.

·       High module temperature reduces electrical performance and changes the expected output.

·       A simple energy graph shows that production changed, but it may not explain why.

·       Cleaning too early wastes water and labor, while cleaning too late loses energy.

·       A hardware fault can remain hidden if it looks similar to normal weather variation.

The data reproduced in the reviewed study show that PV efficiency decreased from 13.88% for the clean panel to 1.83% at a dust density of 5.29 mg/cm² [2].

# Proposed solution and objectives

Aegis-PV is designed as a virtual solar technician for residential users. It does more than display power. It builds an expected-output baseline, measures the gap between expected and actual production, then studies the shape of that gap over time. The system returns a likely diagnosis and a practical next step.

## Project objectives

1.          Estimate the output that the solar system should produce under current weather and system conditions.

2.          Detect unusual differences between expected output and actual inverter readings.

3.          Classify the likely cause as soiling, shading, thermal loss, or equipment failure.

4.          Give the homeowner one clear action with a confidence score and priority level.

5.          Support different inverter brands through standard Modbus or REST interfaces.

## Core user value

| Visibility | Shows the size and timing of the performance gap. |
| --- | --- |
| Diagnosis | Explains the likely cause instead of only showing a low-output alert. |
| Action | Suggests cleaning, shade inspection, wiring checks, or service support. |
| Local fit | Plans to adjust the baseline for GCC dust and temperature conditions. |
| Access | Targets non-technical homeowners with simple language and clear priorities. |

The proposal is intentionally focused on decision support. Aegis-PV should not claim that every diagnosis is certain. It should show the confidence level, the evidence behind the alert, and what the user can safely check before requesting technical service.

# System architecture

The planned platform uses a four-stage pipeline. Each stage has a separate role, which makes the system easier to test, improve, and connect to different solar installations.

| 1 Data Ingestion | 2 Baseline Model | 3 AI Diagnosis | 4 User Action |
| --- | --- | --- | --- |
| Weather, inverter data, and system details | Digital twin estimates expected output | Models find and classify unusual losses | Dashboard explains the cause and next step |

## Data ingestion layer

The platform collects three groups of inputs. Climate data includes solar irradiance, ambient temperature, and humidity. Operational data includes DC and AC voltage, string current, inverter power, and energy. Context data includes panel tilt, azimuth, array size, location, and manufacturer specifications.

## Processing and storage

A normalization service converts data from different inverter brands into one internal format. Quality checks mark missing values, impossible readings, and time gaps. Clean time-series data is then stored for baseline calculation, diagnosis, dashboard views, and later model training.

## Integration approach

Modbus can support direct communication with compatible devices on the local network. REST APIs can support vendor cloud services or gateway devices. The first prototype should use a small connector layer so each new inverter requires only a mapping file and a tested adapter, rather than changes to the full analytics system.

## Security and privacy

The platform should use encrypted connections, strong device identity, and role-based access. It should collect only the data needed for diagnosis. The design should separate homeowner information from technical telemetry and should keep an audit record of alerts and recommendations.

# Digital twin and AI diagnosis

The digital twin is a software model of the home solar system. It estimates how much power the system should produce at a given time. The estimate uses available sunlight, panel area, panel efficiency, orientation, and temperature effects. The result is called the expected-output baseline.

## Gap analysis

Aegis-PV compares actual inverter power with expected power. A small short-term gap may be normal. A large or repeated gap may indicate a problem. The analysis should consider time of day, recent weather, data quality, and the length of the loss before issuing an alert.

## Planned model roles

| Baseline model | Calculates expected production after weather and temperature adjustments. |
| --- | --- |
| Isolation Forest | Flags unusual gaps or operating patterns without needing a label for every event. |
| Random Forest | Classifies a detected event when labeled examples are available. |
| Rules layer | Checks safety limits, missing data, and simple known patterns. |
| Confidence layer | Combines model probability, data quality, and event duration. |

## Example fault signatures

·       Soiling usually creates a gradual and persistent reduction across several days.

·       Fixed shading may create a repeating dip at a similar time each day.

·       A string or inverter fault may create a sudden step down or a complete loss of output.

·       Heat-related loss should track temperature and should be handled by the baseline before fault classification.

These signatures are useful starting points, not proof by themselves. The model must be trained and tested with labeled GCC data. Field validation is necessary because dust type, roof design, panel technology, and microclimate can change the signal.

# Dashboard and maintenance workflow

The dashboard should help a homeowner answer three questions quickly. Is the system healthy? What is causing the loss? What should I do next? Detailed graphs can remain available for installers, but the main screen should use simple language.

## Main dashboard features

| System health score | A 0 to 100 score based on current performance, open faults, and data quality. |
| --- | --- |
| Gap analysis graph | Expected and actual power on the same time line. |
| Fault card | Likely cause, confidence, affected area, first seen time, and severity. |
| Maintenance log | A prioritized list of actions and their status. |
| History | Past alerts, completed work, cleaning dates, and measured recovery. |

## Example user journey

6.          The system sees a steady fall in output while weather conditions remain similar.

7.          The baseline confirms that temperature and irradiance do not explain the full loss.

8.          The diagnostic model marks soiling as the most likely cause with a confidence score.

9.          The user receives a message such as Soiling likely on Array B  Schedule cleaning within seven days.

10.    After cleaning, the platform measures the recovery and closes or updates the alert.

## Alert design

Alerts should have clear thresholds so users do not receive too many messages. The proposal suggests issuing normal diagnostic alerts only above 85 percent confidence. Critical electrical conditions should follow separate safety rules and should direct the homeowner to a qualified technician. Every alert should explain which data supported the decision.

# Competitive comparison

The table below uses the comparison requested for this report. It shows Aegis-PV as a proposed product, so planned features are marked clearly and are not presented as completed capabilities.

| Capability | Enphase or SolarEdge | Applus+ | 3E SynaptiQ | Aegis-PV Proposed |
| --- | --- | --- | --- | --- |
| Fault Detection and Diagnostics | Yes | Yes | Yes | Planned |
| Advanced Analytics or AI | Yes or varies by platform | ML and data science | Advanced analytics and digital twin | Planned ML diagnosis |
| Soiling or Dust Analysis | Limited or system dependent | Soiling rate and loss analysis | Loss and root cause analytics | Core proposed feature |
| Shading or Performance Loss Diagnosis | Some diagnostics available | Performance analytics | Root cause loss analysis | Core proposed feature |
| Hardware Agnostic | No mainly vendor ecosystems | Partial integrates plant or SCADA data | Yes | Planned through Modbus or REST |
| GCC Specific Climate Calibration | Not established in reviewed sources | Not established in reviewed sources | Not established in reviewed sources | Proposed GCC focus |
| Residential Focus | Strong | No mainly professional or utility | No professional asset management | Core target market |
| Actionable Maintenance Guidance | Alerts and troubleshooting vary | Analyst supported insights | Recommendations and task management | Planned homeowner actions |
| Digital Twin | Not core in reviewed sources | Not established for this comparison | Yes | Proposed |
| Typical Positioning | Residential vendor ecosystem | Professional utility scale analytics | Professional renewable asset management | GCC residential solar intelligence |

Interpretation  Aegis-PV aims to combine residential access with advanced diagnosis and GCC calibration. Product capabilities and commercial terms should be verified again before any public market claim.

# Implementation and pilot plan

The recommended path is a focused pilot rather than a full product launch. The team should first prove that the baseline and diagnostic models can separate weather effects from real performance losses in local conditions.

## Phase 1  Data and prototype

·       Select one or two supported inverter data sources.

·       Build the common telemetry format and basic quality checks.

·       Create the first digital-twin baseline using irradiance and temperature.

·       Build a dashboard with expected power, actual power, and a simple health status.

## Phase 2  Labeled GCC pilot

·       Recruit a small group of homes or test arrays in different local conditions.

·       Record cleaning events, shading observations, faults, and technician findings.

·       Train the classifier and compare its output with confirmed field events.

·       Measure false alerts, missed faults, diagnosis time, and user understanding.

## Phase 3  Product readiness

·       Add more inverter adapters and automated onboarding.

·       Introduce secure user accounts, alert preferences, and maintenance history.

·       Create installer tools for fleet views and case review.

·       Document model limits, data retention, security controls, and support procedures.

## Pilot success measures

| Baseline accuracy | Error between expected and measured output during healthy operation. |
| --- | --- |
| Diagnosis quality | Precision and recall for soiling, shading, and hardware faults. |
| Alert quality | False alerts per home and percentage of alerts that lead to useful action. |
| Energy recovery | Measured improvement after a recommended maintenance action. |
| User clarity | Percentage of users who understand the alert and the next step. |

# Impact business case and risks

## Expected impact

Aegis-PV can improve energy yield when it helps users find losses earlier and choose the correct maintenance action. Targeted cleaning may also reduce unnecessary water and labor use. Early hardware diagnosis can shorten downtime and prevent a small problem from becoming a larger repair. These benefits must be measured during the pilot rather than assumed from laboratory results.

## Business approach

The first customers could be residential solar installers, housing developers, and property managers. A subscription could include monitoring, diagnosis, and maintenance records. Installers may value a fleet view that helps them prioritize visits. The residential interface should remain simple, while professional users can access deeper technical evidence.

## Main risks and controls

| Different inverter data | Use a common data model, connector tests, and clear data-quality flags. |
| --- | --- |
| Limited local labels | Start with rules and anomaly detection, then train classifiers from verified pilot events. |
| Microclimate variation | Validate in several GCC locations and retrain only after review. |
| False alert fatigue | Use confidence thresholds, persistence checks, and alert grouping. |
| Unsafe user action | Separate safe cleaning advice from electrical work that requires a technician. |
| Cybersecurity | Encrypt connections, protect credentials, log access, and minimize stored personal data. |

## Key dependencies

The project depends on access to reliable weather data, inverter telemetry, panel specifications, and verified examples of real faults. Partnerships with local installers can provide these inputs and can also confirm whether the recommendations are practical in the field.

# Conclusion and next decisions

Aegis-PV is a strong hackathon concept because it solves a practical problem with a clear technical method. The proposal connects a digital twin with anomaly detection and fault classification, then turns the result into a maintenance action. Its proposed focus on GCC conditions and residential users gives the project a clear position.

The concept is still at the proposed stage. The most important proof will come from local pilot data. The team should avoid claiming that the model can already diagnose every fault or work with every inverter. A credible next version should show one complete path from live data to a verified diagnosis and measured recovery.

## Recommended next decisions

1.  Choose the first inverter or gateway integration and confirm access to its data.

2.  Define the minimum set of inputs required for a reliable expected-output baseline.

3.  Secure a pilot partner and a process for labeling cleaning, shading, and fault events.

4.  Set measurable acceptance targets for baseline error, diagnosis quality, and false alerts.

5.  Build a homeowner alert flow that separates safe actions from technician-only work.

## References

[1] Team SMANERGY. GSR Hackathon Project Brief. Aegis-PV AI-Driven Predictive Health and Performance Maximizer. 2026.

[2] The Effect of Dust Deposition on the Performance of Photovoltaic Panels. Energies 16, 6794. https://www.mdpi.com/1996-1073/16/19/6794

[3] The Impact of Soiling on PV Module Performance in Saudi Arabia. Energies 15, 8033. https://www.mdpi.com/1996-1073/15/21/8033

[4] Saudi Vision 2030. Saudi Green Initiative. https://www.vision2030.gov.sa/en/explore/projects/saudi-green-initiative

Report scope  Proposed functions are described as planned features. Results from published studies should be validated for the final pilot location and system design.